| Test Procedure | EV-SOMCRR-EZLITE |
| --- | --- |

Document No.:	18-070361-01 Rev B

Title:	EV-SOMCRR-EZLITE Customer Evaluation Board Test    Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| Prelim | ECR-????? | Initial Release | 02/15/23 | David Harrington |
| A |  | Release | 03/01/23 | David Harrington |
| B |  | Release | 03/08/23 | David Harrington |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| Apps Engineer |  |

EV-SOMCRR-EZLITE Testing Procedure

| Part Number: | EV-SOMCRR-EZLITE | Description: | System on Module base evaluation board |
| --- | --- | --- | --- |
| Date: | 03/08/2023 | Written by: | David Harrington |
| Test Fixture: | Laptop | SW – Initial/Date: |  |
| Approved by: |  | HW – Initial/Date: |  |
| Computer: |  |  |  |

- Document Revision History

| Revision Number | Date | Description | Engineer |
| --- | --- | --- | --- |
| Prelim | 11/18/2022 | Initial release | David Harrington |
| A | 03/01/2023 | Rev A | David Harrington |
| B | 03/08/2023 | Rev B | David Harrington |

- Required Equipment

  - (1) 3.5MM STEREO Cables

  - (2) 3ft USB 2.0 A MALE TO USB-C MALE CABLE

  - 12V 18W AC/DC External Power Supply P/N: EMSA120150-P5RP-SZ

  - EV-SC598-SOM

  - SAMTEC Linkport Cable

- Required software/files

  - EV-SOMCRR-EZLITE Manufacturing.zip file

  - ADI CCES version 2.11.1  or later

  - Termite Terminal Emulator

- Setup

  - 1 Switch and Jumper Settings

•	Boot mode switch on SoM (SW1) is set to 1

•	JP1 on SOM set to pins 2-3

•	P6 Pins 1&2 (Debug Agent IO Select)

•	P10 OFF (USB Host/Device)

•	S1 1, 3, 5 ON 2,4,6 OFF

•	S2 1, 3 ON 2,4,5,6 OFF

•	SW4 Pins 1 thru 6 ON, Pin 7-8 OFF (JTAG Enable)

  - 2   Connections

- Connect 3.5mm cable to P8 and P7.

- Linkport Cable connected between P3 and P4.

- Programmed Parts

  - Check BOM Revision

Ensure that the BOM REV is the latest.

  - Programming the Debug Agent (U28)

Please use latest Debug Agent Flash Programmer and latest DA Firmware.

- Open the DebugAgentFlashProgrammer utility.

- Click on the “Firmware” tab.

- Under “Targets”, click the “All” box.

- In the “Firmware” drop-down tab, select EV-SOMCRR-EZKIT and click “Flash”.

- In the “Flash targets?” confirmation box, click “Yes”.

- Verify that the following information is displayed.

Power cycle the board to continue. Green D10 should be lit up.

Debug: If U28 fails, you can erase it using a jumper across P17 labeled Erase. Unplug USB-C from P16, place jumper on P17, then plug USB-C in P16. It should be now erased.

  - Programming Serial Flash on the SoM

- Now proceed to the test machine that will be used to test the board.

- Extract all of the files from the previously installed “manufacturing” zip folder to the desktop.

- Make sure the boot mode switch on the SC598 SOM (SW1) is set to 1.

- Connect a USB-C cable from the PC to the USB DA (P16)

- Connect a USB-C from the PC to the USB TO UART connector on the DSP SoM

- Plug 12V power supply into the EZ-Board.

- To load Serial flash (on DSP SoM) with

  - Navigate to the “manufacturing” folder.

  - Double click on the Flash_Programmer_EZLITE-DA.bat file.

  - Make sure there were no errors during programming.

  - Press any key to exit.

- Open a Termite terminal session and configure the serial terminal with the following settings: COMxxx (Not COM3), 9600 BAUD, Data bits: 8, Parity: none, Stop bits: 1, Flow control: None, Append LF selected, Font: monospaced

- Tests

WARNING: Please perform the tests in the order described here to avoid possible damage to the hardware and/or unpredictable results.

Be sure boot mode switch (SW1) is set to 1.

Here is the Tests included in the EV-SOMCRR-EZLITE POST:

| 0 | 3 Time Default Loop |
| --- | --- |
| 1 | 3 Time Standard Loop |
| 2 | Cont Standard Loop |
| 3 | SOM UART Test |
| 4 | SOM DDR3 Test |
| 5 | SOM QSPI Test |
| 6 | SOM eMMC Test |
| 7 | SOM SI5356A Test |
| 8 | SOM TMU Test |
| 9 | Pushbutton Test |
| 10 | ADC DAC Test 1 |
| 11 | ADC DAC Test 2 |
| 12 | Linkport Test |
| 13 | USB QSPI Test |
| 14 | SPI0 MicroSD Test |
| 15 | USB3317 Test |
| 16 | EMAC0 ADIN1300 Test |
| 17 | Toggle_Loop_Flag |
| 18 | Display POST Status |

Stand Alone One-time tests

ADC/DAC Test 2

- Plug Stereo cable in between P8 and P7. In the Termite terminal text window, type 11 and press enter and verify that test passed.

Standard Loop Tests (3 Loops)

- Plug Stereo cable in between P8 and P5.

- Press the Reset button (S3) on the SC598 SOM. DS9 will light when reset is hit.

- In the Termite terminal text window, type 1 and press enter.

- Follow the instructions in the Termite window and press PB1 and PB2.

- The Standard Loop tests will continue, without user input for 3 loops.

- Verify that there were no failures when the POST stats are displayed at the end of the 3 loops.

When you are finished the standard loop test you may power off the EV-SOMCRR-EZLITE and remove the EV-SC598-SOM.