##### EVAL-AD7746 Test Procedure

### Software setup

N.B. Install the AD7746 Test Procedure software before connecting the AD7746 Evaluation board to the PC

- Place the AD7746 Test procedure CD ROM in the CD-ROM drive of the PC and follow the instructions that appear automatically.

### Setting up the board

- Connect the board to the USB port of the PC via the USB Cable.

- The PC will detect that the board has been connected.  (This may take a few seconds).  A message should appear stating that the AD774x Evaluation Board has been found.

- Click ‘Next’

- During the installation, if the following window appears:

- Click ‘Continue Anyway’. And then click Finish.

Note: this procedure will only be present the first time you connect the board to your PC, this will not occur for any subsequent board tests.

Running The AD7746 Evaluation Board Software

Run the AD7746 Evaluation Board Software by selecting it form:

Program Files ->Analog Devices-> AD7745-46 Evaluation Software

The following window will appear:

Test 1 – Running the Capacitive Length Sensor Demo:

- Hit the LENGTH DEMO (CH2) button at the top of the screen.

- This enables the capacitive length sensor demo.

- Move the ruler on the the AD7745/46 Evaluation Board.

- Verify that this action is verified onscreen. The value in mm and hex should also change as the ruler is moved in and out of the slot.

Test 2 Testing Capacitive channel 1.

- hit BACK to return to the main menu.

- Hit the QUICK SETUP (CH1) button

- Connect one end of the supplied SMB Block Capacitance to the SMB connector labeled CIN1(+) and connect the other end to the SMB connector labeled EXCB.

- Hit START

- Verify that the value in the capacitance dialog box reads approximately 4 pF.

- Remove the end of the SMB block capacitance that is connected to CIN1+ and connect it to CIN1-.

- Verify that the value in the capacitance dialog box reads approximately - 4pF.

When you have finished with a board, disconnect it from the PC and connect the new board to be tested. Hit the Download Firmware button and start with tests 1 and 2. Please complete tests 1 and 2 for all boards.

If at any stage the software causes the PC to lock up, disconnect the AD7746 Evaluation board form the PC. When a dialog box appears informing you of a ‘Connection Failure’, hit OK, then reconnect the board and hit Download Firmware and start again.

Evaluation Kit Contents

Fully assembled/tested Eval-AD7746 PCB

Anti-Static bag

USB cable

Bar Code Box Label