This anomaly sheet describes the known bugs, anomalies, and workarounds for the ADF7023-J transceiver and relates to Silicon Revision 1.2, which has corresponding silicon revision readback codes of 0x70, 0x23, 0x01, and 0x02. See the ADF7023-J data sheet for details on how to read the silicon revision codes.

Analog Devices, Inc., is committed, through future silicon revisions, to continuously improve silicon functionality. Analog Devices tries to ensure that these future silicon revisions remain compatible with your present software/systems by implementing the recommended workarounds outlined here.

ADF7023-J Functionality Issues

| Silicon Revision Readback | Chip Marking | Silicon Status | Anomaly Sheet | No. of Reported Anomalies |
| --- | --- | --- | --- | --- |
| Product Code MSB = 0x70 Product Code LSB = 0x23 Silicon Revision Code MSB = 0x01 Silicon Revision Code LSB = 0x02 | ADF7023-JBCPZ | Release | Rev. 0 | 5 |

## Functionality Issues

Table 1. RC Oscillator Accuracy [er001]

| Background | The RC oscillator can be used to control the wake-up timing of the ADF7023-J low power modes. Its typical accuracy is specified as 1.5% after a calibration at 25°C. |
| --- | --- |
| Issue | The calibration range of the RC oscillator is not sufficient to ensure a 1.5% typical accuracy on all devices. This can result in the postcalibration accuracy on some devices being significantly greater than 1.5%. |
| Workaround | For applications requiring accurate low power mode timing, the 32.768 kHz external oscillator should be used. |
| Related Issues | None. |

Table 2. External PA and LNA Enable on ADCIN_ATB3 and ATB4 [er002]

| Background | External PA and LNA enable signals from the ADF7023-J can be configured on the ADCIN_ATB3 and ATB4 pins, respectively. The specified typical VOH is 1.8 V and the specified typical VOL is 0.1 V. |
| --- | --- |
| Issue | The output voltage levels of the PA and LNA enable signals on ADCIN_ATB3 and ATB4 do not meet the specified VOH and VOL specifications in the ADF7023-J data sheet. The actual VOH and VOL levels are shown in Table 3. |
| Workaround | An external level translator or buffer may be required to ensure that the appropriate logic trigger points of the external circuitry are met. |
| Related Issues | er003 (see Table 4). |

Table 3. Typical Output Voltage Levels on ADCIN_ATB3 and ATB4 [er002]

| Parameter | Typical (V) | Conditions |
| --- | --- | --- |
| ADCIN_ATB3, Output High Voltage, VOH | 2.2 | VDD = 3.6 V, IOH = 100 µA |
| ADCIN_ATB3, Output Low Voltage, VOL | 0.64 | VDD = 3.6 V, IOL = 0.2 µA |
| ADCIN_ATB3, Output Low Voltage, VOL | 2.04 | VDD = 3.6 V, IOL = 1.0 µA |
| ATB4, Output High Voltage, VOH | 2.2 | VDD = 3.6 V, IOH = 100 µA |
| ATB4, Output Low Voltage, VOL | 0.53 | VDD = 3.6 V, IOL = 0.2 µA |
| ATB4, Output Low Voltage, VOL | 2.08 | VDD = 3.6 V, IOL = 1.0 µA |
| ADCIN_ATB3, Output High Voltage, VOH | 1.38 | VDD = 1.8 V, IOH = 100 µA |
| ADCIN_ATB3, Output Low Voltage, VOL | 0.65 | VDD = 1.8 V, IOL = 0.2 µA |
| ADCIN_ATB3, Output Low Voltage, VOL | 1.8 | VDD = 1.8 V, IOL = 1.0 µA |
| ATB4, Output High Voltage, VOH | 1.38 | VDD = 1.8 V, IOH = 100 µA |
| ATB4, Output Low Voltage, VOL | 0.55 | VDD = 1.8 V, IOL = 0.2 µA |
| ATB4, Output Low Voltage, VOL | 1.8 | VDD = 1.8 V, IOL = 1.0 µA |

Table 4. External PA and LNA Enable on XOSC32KP_GP5_ATB1 and XOSC32KN_ATB2 [er003]

| Background | External PA and LNA enable signals from the ADF7023-J can be configured on the XOSC32KP_GP5_ATB1 and XOSC32KN_ATB2 pins. |
| --- | --- |
| Issue | The PA and LNA enable signals on XOSC32KP_GP5_ATB1 and XOSC32KN_ATB2 are nonfunctional. |
| Workaround | Use the external PA and LNA enable signals on ADCIN_ATB3 and ATB4. |
| Related Issues | er002 (see Table 2). |

Table 5. Optimum Uncalibrated Image Attenuation [er004]

| Background | The typical uncalibrated image attenuation is specified as 36 dB at 954 MHz. |
| --- | --- |
| Issue | To achieve the typical uncalibrated image attenuation values specified in the ADF7023-J data sheet, it is required to use the recommended default values for IMAGE_REJECT_CAL_PHASE (Address 0x118) and IMAGE_REJECT_CAL_AMPLITUDE (Address 0x119). |
| Workaround | To achieve the specified uncalibrated image attenuation at 954 MHz, set IMAGE_REJECT_CAL_AMPLITUDE = 8 and IMAGE_REJECT_CAL_PHASE = 55. |
| Related Issues | None. |

Table 6. Receiver Sensitivity at 910 MHz [er005]

| Background | The 910 MHz channel falls exactly on the 35th harmonic of the 26 MHz crystal reference. |
| --- | --- |
| Issue | When the ADF7023-J receiver is configured for reception at 910 MHz ± (IF bandwidth × 2), the receiver sensitivity can be degraded by up to 20 dB. |
| Workaround | The degradation in receiver sensitivity at 910 MHz ± (IF bandwidth × 2) can be significantly improved by using the following configuration: - Disable the AGC by setting AGC_LOCK_MODE (in Register RADIO_CFG_7, Address 0x113) = 0x01. - Set the receiver gain to the maximum by setting AGC_MODE (Address 0x35D) = 0x36. - Disable the ADC by writing 0x0F to MCR Register 0x324. |
| Related Issues | None. |

## Section 1. ADF7023-J Functionality Issues

| Reference Number | Description | Status |
| --- | --- | --- |
| er001 | RC oscillator accuracy | Open |
| er002 | External PA and LNA enable on ADCIN_ATB3 and ATB4 | Open |
| er003 | External PA and LNA enable on XOSC32KP_GP5_ATB1 and XOSC32KN_ATB2 | Open |
| er004 | Optimum uncalibrated image attenuation | Open |
| er005 | Receiver sensitivity at 910 MHz | Open |

Notes

| ©2011 Analog Devices, Inc. All rights reserved. Trademarks and registered trademarks are the property of their respective owners. S09834-0-4/11(0) |  |
| --- | --- |